BACKHOE  KINEMATICS & DYNAMICS

For use in the design of the Haptic Backhoe project

Joe Frankel

Georgia Institute of Technology

Intelligent Machines Dynamics Laboratory

August 20, 2003

CONTENTS

Abstract

Component Definitions and Labeling Conventions

Cylinder Space to Joint Space Transformation

Forward Displacement Analysis

Reverse Displacement Analysis

Joint Space to Cylinder Space Transformation

Actuator Force to Joint Torque Transformation

LaGrangian Dynamics

Bucket Trajectory Simulation

References

Appendix

Mass and Inertia Properties from ProENGINEER

Dynamic Model Elements from Matlab

Abstract

This report is intended for use in the design of the Robotic Backhoe with Haptic Display project at the Intelligent Machine Dynamics Laboratory (IMDL) at Georgia Tech. The work herein describes the kinematic and dynamic relationships between the cylinder forces and resulting motions of the backhoe’s four degrees of freedom, namely the swing, boom, stick, and bucket links. The geometric transformations between joint angles and cylinder positions are derived, and both forward and reverse displacement analyses are solved. A LaGrangian dynamic model is presented to compute link accelerations from actuator forces. Finally, a desired digging trajectory is defined, and the reverse kinematic equations are used for simulation. This report is intended to be used as a stepping stone for programming the controller and haptic user interface for the Robotic Backhoe with Haptic Display.

Figure 1 is a photo of the backhoe used for this project, before modifications.

Figure 1: John Deere Model 47 Backhoe

Component Definitions and Labeling Conventions

The backhoe can be modeled as a 4R (four revolute joint) hybrid serial-parallel mechanism, with the swing joint axis normal to the ground and the remaining three joint axes parallel to the ground. Figure 1 is a photo of the backhoe, and Figure 2 illustrates the component frames and joint angles as used throughout this report. Note that relevant points in the mechanism have been labeled with numbers O1-O4 for the joint axis origins and letters A-H for the remaining axes relevant to the analysis. When describing linear and angular dimensions, the following convention has been ascribed. All linear (scalar) dimensions are given as rxy, where x and y are either the numeric or alphabetic joint labels, with numbers before letters, and arranged in ascending order. For example, the distance from point B to origin O1 would be r1B, etc. Angular quantities are given in the format xyz, where x, y, and z are the three points describing the angle in the counterclockwise direction. For example, the angle between the lines O2O3and EO3is given by 23E. See Figure 2.

Figure 2: Component Frames and Joint Angles

Scalar quantities are shown in italics. For example, the swing angle is given by 1. Vector and matrix quantities are shown in boldface format, with one-dimensional column vectors in lower case and two-dimensional matrices in uppercase. For example, θ=θ1θ2θ3θ4Tindicates the four element joint angle vector and M is the 4x4 manipulator inertia matrix.

Position vectors are indicated in the format pxyf, where the superscript f denotes the component frame and the subscript xy denotes the starting and ending points. For example, the notation pC32 denotes the position vector from point C to origin O3 in component frame 2 (boom coordinates).

Cylinder lengths are denoted by the vector yc, where yc(i), i=1:5. Although the backhoe has four degrees of freedom, the swing link is associated with the first two cylinders, so yc(3) controls the boom, yc(4) the stick, and yc(5) the bucket. See Figure 2.

Some of the kinematic transformations make use of homogeneous transformation matrices and the geometric Jacobian. For these computations, the Denavit-Hartenberg labeling (DH) convention is used. The DH parameters for the backhoe are given in Table 1.  The joint angles  uniquely determine the pose of the backhoe. The link lengths a and the twist angles  are constants.

Table 1: Denavit-Hartenberg Parameters

| Joint | Link Lengths a(i) | Joint Angles (i) | Joint Offsets d(i) | Twist Angles (i) |
| --- | --- | --- | --- | --- |
| 1 | a1 | 1 | 0 | 90 |
| 2 | a2 | 2 | 0 | 0 |
| 3 | a3 | 3 | 0 | 0 |
| 4 | a4 | 4 | 0 | 0 |

The bucket angle  has been defined as the angle between the x4 axis (bucket) and the x0-y0 plane, which is assumed to be parallel with the earth. The bucket angle is defined as positive downward as illustrated in Figure 3. This angle will be useful for describing the desired bucket trajectory in base frame coordinates.

Finally, subscripts and indices in parenthesis are used interchangeably in some cases. For example, 1=(1), etc.

Figure 3: Bucket Angle 

x0

x0-y0 plane

parallel with the earth

z0

+ 

- 

Bucket angle

Cylinder Space to Joint Space Transformation

cyl_to_joint

function

The following section derives the equations necessary to compute the joint angles from the cylinder lengths by analyzing each link independently from base to tip. All quantities are either known constants or functions of the input variables yc(i), and the equations represent the dynamic quantities that must be computed at each simulation step. All of the constant lengths and angles are either stored in memory or computed offline beforehand and have been omitted below for brevity. It is assumed that the cylinder lengths will be available from suitable linear position sensors mounted to the backhoe.

Figure 4: Link 1 Transformation

yc(1)

O0

O1

K

J

M

x1

Q

x0

z1

y0

Link 1

(swing)

Link 0

(base)

yc(2)

1

1

J’

Q’

Figure 4 is a sketch of the base (tractor) and swing links as viewed from above.

The transformation from the known cylinder length yc(1) to the joint angle 1 is computed as follows.

θK0J=cos−1r0K2+r0J2−yc212r0Kr0J				(1)

θ1=θK0J−θK0Q−θMJ0					(2)

Note that in practice, the angle K0J  must be checked to see whether it is greater than 180, which requires knowledge of yc(2) as well.

Figure 5: Link 2 Transformation

A

B

C

O1

O2

x1

y1

x2

y2

yc(3)

2

Link 2

(boom)

Link 1

(swing)

Figure 5 is a sketch of the swing and boom links as viewed from the side. The transformation from the boom cylinder length yc(3) to the joint angle 2 is as follows.

θA1B=cos−1r1A2+r1B2−yc232r1Ar1B				(3)

θ2=π−θ01A−θA1B−θB12					(4)

Figure 6: Link 3 Transformation

yc(4)

Link 2

(boom)

Link 3

(stick)

C

D

O2

x2

x3

3

y3

Figure 6 is a sketch of the boom and stick links as viewed from the side. The transformation from the stick cylinder length yc(4) to the joint angle 3 is as follows.

θC2D=cos−1r2C2+r2D2−yc242r2Cr2D				(5)

θ3=3π−θ12C−θC2D−θD23					(6)

Figure 7 is a sketch of the stick and bucket links as viewed from the side. The transformation from the boom cylinder length yc(5) to the joint angle 4 is as follows.

Figure 7: Link 4 Transformation

Link 3

(stick)

Link 4

(bucket)

O3

D

E

yc(5)

G

H

F

O4

x4

y4

x3

4

O2

θEFH=cos−1rEF2+rFH2−yc252rEFrFH				(7)

θHF3=π−θDFE−θEFH					(8)

r3H=r3F2+rFH2−2r3FrFHcosθHF3				(9)

θF3H=cos−1r3F2+r3H2−rFH22r3Fr3H					(10)

θH3G=cos−1r3H2+r3G2−rGH22r3Hr3G					(11)

θ4=3π−θF3H−θH3G−θG34−θ23D				(12)

Thus with knowledge of the cylinder lengths yc, the joint angles  can be calculated from equations 1-12.

Forward Displacement Analysis

FDisp

function

Using the Denavit-Hartenberg parameters listed in Table 1, the position and orientation of each component frame of the backhoe can be computed from the known joint angles i, i=1:4 by the successive multiplication of homogeneous transformations of the following form [1]:

Aii−1=cosθi−sinθicosαisinθisinαiaicosθisinθicosθicosαi−cosθisinαiaisinθi0sinαicosαidi0001=Rii−1pi−1,ii−101x31	(13)

where Rii−1 is the 3x3 rotation matrix from frame i-1 to frame i and pi−1,ii−1 is the position vector of origin i with respect to origin i-1 expressed in the i-1 component frame.

The position and orientation of the end effector (ie bucket tip) is then computed from

B=A10A21A32A43=R40p0,4001x31				(14)

where B is termed the bucket displacement matrix. The rotation matrix R40can be interpreted as a matrix of projections of the base frame unit vectors onto bucket frame unit vectors,

R40=x0⋅x4x0⋅y4x0⋅z4y0⋅x4y0⋅y4y0⋅z4z0⋅x4x0⋅y4z0⋅z4				(15)

and p0,40=p0,40xp0,40yp0,40zT is the 3x1 position vector of the bucket tip relative to the base frame, expressed in base frame coordinates. Figure 8 illustrates the quantities computed by the bucket displacement matrix.

Figure 8: Bucket displacement matrix quantities

x0

z0

x4

y4

x0

z0

x4

y4

y0

z4

O0

O4

Once the bucket displacement matrix is computed, the bucket angle  is then found from the x4 vector relative to the x0-y0 plane. However, since the x1 is constrained to rotate in the x0-y0 plane and y1 is always parallel to z0, the bucket angle is computed from

φ=atan2y1⋅x4,x1⋅x4+π					(16)

Note that in Matlab, the output of the atan2 function is limited to - <  < , so in the case that x1x4 < 0 and y1x4 > 0, the bucket is curled up tight next to the stick, and equation 16 becomes φ=atan2z0⋅x4x0⋅x4−π.

Thus the position and orientation of the bucket can be computed from knowledge of the joint angles by using equations 13, 14, and 16.

Reverse Displacement Analysis

RDisp

function

The reverse displacement analysis (RDA) is used to compute the vector of joint angles  from the bucket tip position p0,40and the bucket angle . Figure 9 illustrates the quantities relevant to the RDA.

Figure 9: Reverse displacement vectors

O2

O0

x1

y1

O3

O4

O0

O1

rWy

rWx

The swing angle 1 is decoupled from the other links and can be computed directly from the x and y components of the bucket position vector p0,40:

θ1=atan2p0,40y,p0,40x					(17)

The remaining three links form a planar arm, and the joint angles are computed as follows. Using the rotation matricesR10 andR40 to express all vectors in frame 1,

p1,31=p0,41−p0,11−p3,41=R10p0,40−a100−R40a400		(18)

whereR40=R10R21R32R43. See Figure 9. The distance and angle formed from O1 to the wrist O3 is

r13=p1,31x2+p1,31y2=rWx2+rWy2		(19)

θ31x1=tan−1p1,31yp1,31x=tan−1rWyrWx				(20)

Now that all the sides of the triangle O1O2O3 are known, the interior angles can be found using the cosine law,

θ321=cos−1a22+a32−r1322a2a3 					(21)

θ213=cos−1a22+r132−a322a2r13 					(22)

and then the angles 2 and 3 can be found from

θ2=θ31x1+θ213						(23)

θ3=π+θ321							(24)

Figure 10: 4 Geometry

x1

x2

x3

x4

2

3

4





Figure 10 illustrates the relationship between the bucket angle and the joint angles 2, 3 and 4. The last angle 4 is then found from the bucket angle , where

2π−θ2+θ3+π+φ=θ4

or

θ4=φ−θ2−θ3+3π						(25)

joint_to_cyl

function

Joint Space to Cylinder Space Transformation

The joint space to cylinder space transformation is used to compute the cylinder lengths yc from the joint angles . Combined with the reverse displacement algorithm, these routines are useful to compute the required cylinder length time histories for control purposes.

The lengths of the swing cylinders yc(1) and yc(2) are computed from 1 as follows.

θQ0J=tan−1r0MrJM+θ1					(26)

θK0J=θQ0J+π2−θ10K					(27)

yc1=r0J2+r0K2−2r0Jr0KcosθK0J			(28)

θJ'0Q'=tan−1r0MrJM−θ1					(29)

θJ'0K'=θJ'0Q'+π2−θ10K					(30)

yc2=r0J2+r0K2−2r0Jr0KcosθJ'0K'			(31)

Refer to Figure 4 for equations 26-31.

The boom cylinder length is found by rearranging equations 3 & 4:

θA1B=π−θ01A−θ2−θB12					(32)

yc3=r1A2+r1B2−2r1Ar1BcosθA1B			(33)

Refer to Figure 5 for equations 32 & 33.

The stick cylinder length is found by rearranging equations 5 & 6:

θC2D=π−θD23−θ12C−θ3					(34)

yc4=r2C2+r2D2−2r2Cr2DcosθC2D			(35)

The bucket cylinder length is found from four angle additions and four cosine laws. Refer to Figure 7 for equations 36-43.

θx33G=2π−θG34−θ4						(36)

θG3F=π−θx33G+θ23D					(37)

rFG=r3F2+r3G2−2r3Fr3GcosθG3F 			(38)

θ3FG=cos−1r3F2+rFG2−r3G22r3FrFG 				(39)

θHFG=cos−1rFG2+rFH2−rGH22rFGrFH 				(40)

The angle HF3 depends on whether θ4+θG34>2π:

if θ4+θG34>2π θHF3=θHFG+θ3FGotherwise θHF3=θHFG−θ3FG 						(41)

then

θEFH=π−θDFE+θHF3 					(42)

yc5=rEF2+rFH2−2rEFrFHcosθEFH 		(43)

Actuator Force to Joint Torque Transformation

Fr2taur

function

When simulating the backhoe’s response to forces applied to the links by the hydraulic actuators, a transformation is necessary to convert from cylinder rod forces to joint torques. Formally, this step would involve replacing the rod forces fr with a combination of both equivalent couple moments and forces. However, since we are only interested in the torque applied to the joints, the internal forces that intersect the joint axes will be neglected. Note that the magnitude of the applied joint torques r depends on both the rod forces fr and the cylinder lengths yc.

The first joint torque r(1) at O0 about the z0  is computed from the contribution of the two swing cylinder forces fr(1) and fr(2). Figures 11 and 12 illustrate the transformation.

Figure 11: Swing cylinder transformation dimensions

yc(1)

O0

O1

K

J

M

x1

Q

x0

z1

y0

Link 1

(swing)

Link 0

(base)

yc(2)

1

1

J’

Q’

K’

Figure 12: Swing cylinder force to joint 1 torque transformation

O0

O1

x1

x0

z1

y0

O0

x1

x0

z1

y0

ra

rb

rJQ=r0J2+p1K132−2r0Jp1K13cosθQ0J		(44)

θr1=cos−1rQK2+yc21−rJQ22rQKyc1					(45)

rJ'Q'=r0J'2+p1K132−2r0Jp1K13cosθJ'0Q' 		(46)

θr2=2π−cos−1rQK2+yc22−rJ'Q'22rQKyc2 			(47)

p0K1=rQK0p1K13Tp0K'1=rQK0−p1K13T					(48)

fr11=fr1cosθr10sinθr1Tfr21=fr2cosθr20sinθr2T				(49)

τr1=p0K1×fr11+p0K'1×fr21⋅z0				(50)

Note that the five-element cylinder force vector fr does not contain the direction information of the swing cylinder force vectors fr11 and fr21 used in equations 49-50; i.e.

fr=fr1fr2fr3fr4fr5T			 (51)

fr=frswing1frswing2frboomfrstickfrbucketT	(52)

which is related to equation 49 only by the magnitude of the first two elements.

Figures 13 and 14 illustrate the transformation of the force applied by the boom cylinder fr3 into the applied torque r(2) at O1 about z1.

Figure 13: Boom cylinder transformation dimensions

A

B

C

O1

O2

x1

y1

x2

y2

yc(3)

2

Figure 14: Boom cylinder force to joint 2 torque transformation

B

O1

O2

x2

y2

B

O1

O2

x2

y2

r3

θ1BA=cos−1yc23+r1B2−r1A22yc3r1B				(53)

θr3=2π+θB12−θ1BA					(54)

p1B2=r1BcosθB12sinθB120T				(55)

fr32=fr3cosθr3sinθr30T				(56)

τr2=p1B2×fr32⋅z1						(57)

Figures 15 and 16 illustrate the transformation of the force applied by the stick cylinder fr4 into the applied torque r(3) at O2 about z2.

Figure 15: Stick cylinder transformation dimensions

yc(4)

C

D

O2

x2

x3

y3

x’3

Figure 16: Stick cylinder force to joint 3 torque transformation

O3

D

x3

O2

y3

O3

D

x3

O2

y3

r4

r(3)

θ2DC=cos−1r2D2+yc24−r2C22r2Dyc4				(58)

θx'32D=π−θD23						(59)

θr4=π−θ2DC−θx'32D						(60)

p2D3=r2DcosθD23sinθD230T				(61)

fr43=fr4cosθr4sinθr40T				(62)

τr3=p2D3×fr43⋅z2 						(63)

Figure 17 illustrates the points of interest in computing the torque applied to the bucket. Knowledge of the force exerted by the bucket cylinder fr5 and the length of the bucket cylinder yc(5) is required for the transformation.

Figure 17: Bucket cylinder transformation dimensions

O3

G

H

F

O4

x4

y4

x3

4

sublink GH

sublink FH

This transformation is solved by first analyzing the angles of the bucket sublinks FH and GH relative to the stick and bucket, summing forces at the pin connection at H, and finally solving simultaneously for the magnitude of the orthogonal components of the force fG4 that link GH exerts on the bucket at point G.

Figures 18 illustrates the transformation of the force fG4 exerted by the sublink GH onto the bucket, and the corresponding applied torque r(4) at O3 about z3.

Figure 18: Bucket cylinder force to joint 4 torque transformation

x4

O3

G

O4

y4

x4

G

O3

G

O4

y4

r(4)

x4

Figure 19(a) illustrates the geometry of the sublink FH relative to the stick. Figure 19(b) illustrates the forces applied to the pin at point H.

Figure 19: Sublink FH forces and geometry

F

H’

F

x3

O3

y3

x3

x4

r5

F

H

(a)

(b)

The first step is to find the angles G, r5, and F:

θEFH=cos−1rEF2+rFH2−yc252rEFrFH				(64)

θHF3=π−θDFE−θEFH					(65)

r3H=r3F2+rFH2−2r3FrFHcosθHF3				(66)

θF3H=cos−1r3F2+r3H2−rFH22r3Fr3H					(67)

θH3G=cos−1r3H2+r3G2−rGH22r3Hr3G					(68)

θ4=3π−θF3H−θH3G−θG34−θ23D				(69)

θx33G=2π−θ4−θG34						(70)

The angle G3F depends on whether θ4+θG34>2π:

if θ4+θG34>2π θG3F=π−θx33G−θ23Dotherwise θG3F=π−θx33G+θ23D			 		(71)

rFG=r3F2+r3G2−2r3Fr3GcosθG3F			(72)

θFG3=cos−1rFG2+r3G2−r3F22rFGr3G					(73)

θFGH=cos−1rFG2+rGH2−rFH22rFGrGH				(74)

The angle 3GH depends on whether θ4+θG34>2π:

if θ4+θG34>2π θ3GH=θFGH−θFG3otherwise θ3GH=θFGH+θFG3						(75)

The angle G is defined as the angle that the two-force link GH makes with x4:

θG=θG34−θ3GH						(76)

θ3FG=cos−1r3F2+rFG2−r3G22r3FrFG 				(77)

θHFG=cos−1rFG2+rFH2−rGH22rFGrFH				 (78)

The angle HF3 depends on whether θ4+θG34>2π:

if θ4+θG34>2π θHF3=θHFG+θ3FGotherwise θHF3=θHFG−θ3FG 						(79)

θ3FH'=π−θHF3						(80)

θF=π−θ3FH'−θ23D+π					(81)

θFHE=cos−1rFH2+yc25−rEF22rFHyc5				(82)

θr5=−θ23D+θHF3−θFHE					(83)

Now that the angles G, F, and r5 are known from equations 76, 81, and 83, a force balance on pin H in the x3 and y3 directions yields

fr5cosθr5+fFcosθF+fGcosθ4+θG=0fr5sinθr5+fFsinθF+fGsinθ4+θG=0		(84)

where the mass of pin H has been neglected. Rearranging (84) into matrix format and solving for the magnitudes of the unknown forces fF and fG,

fFfG=cosθFcosθ4+θGsinθFsinθ4+θG−1−fr5cosθr5−fr5sinθr5		(85)

See Figure 19(b).

The torque r(4) applied to the bucket at O3 about z3 by the force fG is then found from

p3G4=r3GcosθG34sinθG340T				(86)

fG4=fGcosθGsinθG0T 				(87)

τr4=p3G4×fG4⋅z3 						(89)

LaGrangian Dynamics

LaGrange

function

In the field of robotics, a common approach to the dynamic modeling of serial manipulators is to use the principles laid out by Joseph-Louis LaGrange in 1788 in his work entitled the Mécanique analytique [1], [2], [3]. Unlike the Newton-Euler approach, the LaGrangian approach bases its equations of motion on the kinetic and potential energy of the system in terms of a set of generalized coordinates, which are the rotational and translational positions and velocities of the robot’s joints. The advantage to the LaGrangian approach is that there is no need to compute reaction forces at the connections between links, which are typically of no interest to the analysis.

The LaGrangian function is defined as the difference between the kinetic and potential energy of the mechanical system:

L=K−U							(90)

where K is the scalar sum of the kinetic energy of all the links

K=12i=1nvciTmivci+ωiTIiωi					(91)

and U is the scalar sum of the potential energy of all the links

U=−i=1nmigTp0,ci						(92)

In equation 91, vci is the 3x1 translational velocity vector of link i written at the center of mass, and i is the 3x1 rotational velocity vector of link i, both relative to the base frame. The mass matrix mi is the diagonal 3x3 matrix where mi=diag([mi mi mi]), and the lower case has been used to reserve the upper case M for the results of the present formulation. The inertia matrix Ii is the diagonal 3x3 inertia tensor containing the three principle inertias.

In equation 92, the negative sign accounts for the fact that the gravitational vector g=[0 0 –g]T points in the negative z0 direction. Also, both the gravitational vector g and the position vector of the center of mass relative to the base frame p0,ciare expressed in base frame coordinates and the superscripts have been omitted for brevity. The index n is the number of degrees of freedom of the system.

Equation 91 can be rewritten by using the theory of instantaneous screw motion [4] by defining the relationship between the velocities of each link vci and i and the joint rates θ using the geometric Jacobian matrix:

Jvi=Jvi1Jvi2...Jvii00...0				(93)

Jωi=Jωi1Jωi2...Jωii00...0 				(94)

where the elements of the 3 n link Jacobian submatrices Jvi and Jwi are defined by

Jvij=zj−1×pj−1,cij−1 for a revolute jointzj−1 for a prismatic joint			(95)

Jωij=zj−1 for a revolute joint0(3x1) for a prismatic joint 			(96)

and j=1:i. The vector pj−1,cij−1 is the position of the current link’s center of mass relative to the previous link’s origin expressed in the previous link’s coordinates. Note that the complete Jacobian matrix

Ji=JviJωi							(97)

is written separately for each link.

With the Jacobian matrices defined for each link, the linear and angular velocities of each link can be written in the generalized coordinates as

vciωi=JviJωiθ						(98)

Substituting the components of equation 98 into 91 yields

K=12θTi=1nJviTmiJvi+JωiTIiJωiθ 				(99)

The quantity in parenthesis in equation 99 is termed the n n manipulator inertia matrix M, where

M=i=1nJviTmiJvi+JωiTIiJωi 					(100)

so that

K=12θTMθ 							(101)

Now we are ready to form the equations of motions in terms of the generalized coordinates. Substituting 92 and 101 into 90 yields

L=12θTMθ+i=1nmigTp0,ci 					(102)

The equations of motion can now be described by taking the derivative of equation 102 with respect to θ,  θ, and time t:

ddt∂L∂θi−∂L∂θi=τi						(103)

Substituting equation 90 into 103, this becomes

ddt∂K∂θi−∂K∂θi−∂U∂θi=τi 					(104)

where we have used the fact the K is a function of both θ and θ, but U is only a function of θ. The joint torquei is the torque of link i-1 acting on link i along the zi-1 axis. Note thati may be a sum of torques from more than one source, such as from both actuator forces and forces applied to the end effector.

Evaluating the first term in equation 104 yields

ddt∂K∂θi=j=1nMijθj+j=1nk=1n∂Mij∂θkθjθk			(105)

where the manipulator inertia matrix has been expanded into a sum of scalar elements before the product rule has been applied.

Evaluating the second term of equation 104 yields

∂K∂θi=12j=1nk=1n∂Mjk∂θiθjθk					(106)

and evaluating the third term of equation 104 yields

∂U∂θi=−j=1nmjgTJvji						(107)

where the fact that ∂p0,cj∂θi=∂pj−1,cj∂θi=Jvji has been used from equation 95. Substituting equations 105-107 into 104 and combining terms yields the final form of the LaGrangian dynamic equation:

j=1nMijθj+j=1nk=1n∂Mij∂θk−12∂Mjk∂θiθjθk−j=1nmjgTJvji=τi 		(108)

which is valid for each degree of freedom, i=1:n. Each of the n equations of the form given by 61 can be written more compactly in matrix form:

Mθθ+Vθ, {θ+Gθ=τ					(109)

where

V=j=1nk=1n∂Mij∂θk−12∂Mjk∂θiθjθk				(110)

G=−j=1nmjgTJvji 						(111)

and M is given in equation 100. The vector V is called the velocity coupling vector, which contains the velocity-dependent centripetal and coriolis terms, and the vector G is called the vector of gravitational torques. The variables V and G have been left as capitals to be consistent with the literature. Figure 20 is a block diagram of the relevant portion of the backhoe simulation.

Figure 20: LaGrangian dynamics simulation block diagram

LaGrangian dynamic model

FDisp

Bucket position & orientation

Sum of joint torques

Thus the bucket position and orientation can be computed from the applied joint torques, along with the necessary initial joint positions θ and velocities θ at t=0.

The dynamic model given by equation 109 was constructed using CAD software for the mass and inertia properties, and simulation software to compute the dynamic quantities. The elements of the variables M, V, and G given by equations 100, 110, and 111 were manipulated with software and the results are listed in the Appendix along with the mass and inertia properties calculated from the solid modeling software.

References

[1] Sciavicco, L., Siciliano, B., Modeling and Control of Robot Manipulators, 2nd ed. Springer-Verlag,  London, 2001

[2] Tsai, L.W. Robot Analysis. John Wiley & Sons, NY, 1999.

[3] Craig, J.J., Introduction to Robotics. Addison-Wesley Pub. Co., Reading, MA, 1989.

[4] Lipkin, H., Duffy, J., “Sir Robert Stawell Ball and methodologies of modern screw theory”. J Mechanical Engineering Science, Proc Instn Mech Engrs Vol 216 Part C, 2002.

Appendix

Mass and Inertia Properties from ProENGINEER

BASE ASSEMBLY WITH CYLINDERS

----------------------------

VOLUME =  3.2541377e+02  INCH^3

SURFACE AREA =  1.5227593e+03  INCH^2

AVERAGE DENSITY =  2.2985860e-01 POUND / INCH^3

MASS =  7.4799154e+01 POUND

CENTER OF GRAVITY with respect to ASM_DEF_CSYS coordinate frame:

X   Y   Z    -7.9450335e+00  3.2953767e+00  6.6521587e-02  INCH

INERTIA at CENTER OF GRAVITY with respect to ASM_DEF_CSYS coordinate frame:  (POUND * INCH^2)

INERTIA TENSOR:

Ixx Ixy Ixz  2.3229143e+03 -5.2539074e+02  5.7118446e+01

Iyx Iyy Iyz -5.2539074e+02  5.2772512e+03  1.0302013e+01

Izx Izy Izz  5.7118446e+01  1.0302013e+01  5.3568896e+03

BOOM ASSEMBLY WITH CYLINDER

---------------------------

VOLUME =  4.9585884e+02  INCH^3

SURFACE AREA =  3.4438223e+03  INCH^2

AVERAGE DENSITY =  2.1966004e-01 POUND / INCH^3

MASS =  1.0892037e+02 POUND

CENTER OF GRAVITY with respect to ASM_DEF_CSYS coordinate frame:

X   Y   Z    -2.6439663e+01  3.1270699e+00  0.0000000e+00  INCH

INERTIA at CENTER OF GRAVITY with respect to ASM_DEF_CSYS coordinate frame:  (POUND * INCH^2)

INERTIA TENSOR:

Ixx Ixy Ixz  2.0186501e+03  7.6399946e+02  0.0000000e+00

Iyx Iyy Iyz  7.6399946e+02  1.5264085e+04  0.0000000e+00

Izx Izy Izz  0.0000000e+00  0.0000000e+00  1.6083935e+04

STICK ASSEMBLY WITH CYLINDER

-----------------------------

VOLUME =  4.5121342e+02  INCH^3

SURFACE AREA =  3.1935502e+03  INCH^2

AVERAGE DENSITY =  2.1115129e-01 POUND / INCH^3

MASS =  9.5274297e+01 POUND

CENTER OF GRAVITY with respect to ASM_DEF_CSYS coordinate frame:

X   Y   Z    -2.3363210e+01  2.8479407e+00  0.0000000e+00  INCH

INERTIA at CENTER OF GRAVITY with respect to ASM_DEF_CSYS coordinate frame:  (POUND * INCH^2)

INERTIA TENSOR:

Ixx Ixy Ixz  1.3518286e+03 -5.4510614e+02  0.0000000e+00

Iyx Iyy Iyz -5.4510614e+02  2.2033641e+04  0.0000000e+00

Izx Izy Izz  0.0000000e+00  0.0000000e+00  2.2529499e+04

BUCKET ASSEMBLY WITH CYLINDER

-----------------------------

VOLUME =  2.2684190e+02  INCH^3

SURFACE AREA =  1.7929021e+03  INCH^2

AVERAGE DENSITY =  2.8400000e-01 POUND / INCH^3

MASS =  6.4423099e+01 POUND

CENTER OF GRAVITY with respect to ASM_DEF_CSYS coordinate frame:

X   Y   Z     7.7946254e+00  3.3482795e+00 -7.3503549e-04  INCH

INERTIA at CENTER OF GRAVITY with respect to ASM_DEF_CSYS coordinate frame:  (POUND * INCH^2)

INERTIA TENSOR:

Ixx Ixy Ixz  2.0413482e+03 -4.5279092e+02  5.9602768e-01

Iyx Iyy Iyz -4.5279092e+02  6.2054163e+03 -9.1988628e-02

Izx Izy Izz  5.9602768e-01 -9.1988628e-02  5.9272355e+03

Dynamic Model Elements from Matlab

The code that follows is the results of symbolic manipulations from equations 100, 110, and 111, generated using Matlab’s symbolic math toolbox. Note that the joint angles are given in terms of the variable q rather than . The LaGrangian dynamic model is given by

Mqq+Vq, {q+Gq=τ

where the elements of the matrix M and the vectors V and G are given below.

M(1,1) = 1/2*m2*p2x^2+m3*p3z^2+m3*a1^2+m2*p2z^2+m2*a1^2+m1*p1z^2-1/2*I311*cos(2*q(3)+2*q(2))+1/2*m2*p2x^2*cos(2*q(2))-1/2*m2*p2y^2*cos(2*q(2))+I122+1/2*m2*p2y^2+2*m2*cos(q(2))*p2x*a1-2*m2*sin(q(2))*p2y*a1+m4*p4z^2+1/2*m4*a2^2+2*m3*cos(q(2))*a2*a1+1/2*m3*p3x^2+1/2*I422*cos(2*q(4)+2*q(3)+2*q(2))+m3*p3x*a2*cos(q(3))-m3*p3y*a2*sin(q(3))-1/2*m3*p3y^2*cos(2*q(3)+2*q(2))-2*m3*p3y*a1*sin(q(3)+q(2))+m3*p3x*a2*cos(q(3)+2*q(2))-m3*p3y*a2*sin(q(3)+2*q(2))-m3*p3x*p3y*sin(2*q(3)+2*q(2))+2*m3*p3x*a1*cos(q(3)+q(2))+1/2*m3*p3x^2*cos(2*q(3)+2*q(2))+1/2*m3*a2^2*cos(2*q(2))+1/2*m3*p3y^2+1/2*m3*a2^2+1/2*I421*sin(2*q(4)+2*q(3)+2*q(2))-1/2*I411*cos(2*q(4)+2*q(3)+2*q(2))+1/2*I412*sin(2*q(4)+2*q(3)+2*q(2))+1/2*I312*sin(2*q(3)+2*q(2))+1/2*I221*sin(2*q(2))+1/2*I322*cos(2*q(3)+2*q(2))-1/2*I211*cos(2*q(2))+1/2*I222*cos(2*q(2))+1/2*I212*sin(2*q(2))-m2*p2x*p2y*sin(2*q(2))+2*m4*a2*a1*cos(q(2))-1/2*m4*p4y^2*cos(2*q(4)+2*q(3)+2*q(2))+1/2*m4*p4y^2+m4*a1^2+1/2*I411+1/2*I422+1/2*I211+1/2*I222+1/2*m4*p4x^2+1/2*m4*a3^2+1/2*m4*a3^2*cos(2*q(3)+2*q(2))+1/2*m4*p4x^2*cos(2*q(4)+2*q(3)+2*q(2))-m4*p4y*a3*sin(q(4))+m4*a3*a2*cos(q(3))+2*m4*a3*a1*cos(q(3)+q(2))+m4*a3*a2*cos(q(3)+2*q(2))+m4*p4x*a2*cos(q(4)+q(3)+2*q(2))+m4*p4x*a3*cos(q(4)+2*q(3)+2*q(2))-m4*p4y*a2*sin(q(4)+q(3))+m4*p4x*a3*cos(q(4))+1/2*m4*a2^2*cos(2*q(2))-m4*p4x*p4y*sin(2*q(4)+2*q(3)+2*q(2))+m4*p4x*a2*cos(q(4)+q(3))+2*m4*p4x*a1*cos(q(4)+q(3)+q(2))-m4*p4y*a2*sin(q(4)+q(3)+2*q(2))-2*m4*p4y*a1*sin(q(4)+q(3)+q(2))-m4*p4y*a3*sin(q(4)+2*q(3)+2*q(2))+1/2*I321*sin(2*q(3)+2*q(2))+m1*p1x^2+1/2*I311+1/2*I322

M(1,2) = .2e-7*cos(q3)+.304834872174*sin(q2+q3+q4)-.250872124292e-1*cos(q2+q3+q4)-.4e-8*sin(2*q1)-.27e-8*cos(2*q1)+.4e-8*sin(2*q2+2*q3)+.520e-7-.3e-7*cos(2*q3+2*q1+2*q2)+.1e-6*cos(2*q1-2*q2)-.4e-8*sin(2*q2)-.250872124292e-1*cos(1.*q4+1.*q3+1.*q2)+.304834872174*sin(1.*q4+1.*q3+1.*q2)-.1e-6*cos(2*q1+2*q2)+.87e-10*sin(2*q1+q4+q3+q2)+.2e-7*cos(q3+2*q2)

M(1,3) = .304834872174*sin(q2+q3+q4)-.250872124292e-1*cos(q2+q3+q4)+.4e-8*sin(2*q1)+.94e-8*cos(2*q1)-.47e-8*cos(2*q2+2*q3)-.2e-8*sin(2*q2+2*q3)-.250872124292e-1*cos(1.*q4+1.*q3+1.*q2)+.304834872174*sin(1.*q4+1.*q3+1.*q2)

M(1,4) = I423*cos(q(4)+q(3)+q(2))+I413*sin(q(4)+q(3)+q(2))-m4*p4z*sin(q(4)+q(3)+q(2))*p4x-m4*p4z*cos(q(4)+q(3)+q(2))*p4y

M(2,1) = -m3*p3z*sin(q(3)+q(2))*p3x-m3*p3z*cos(q(3)+q(2))*p3y+cos(q(2))*I232+I431*sin(q(4)+q(3)+q(2))+sin(q(2))*I231-m2*p2z*sin(q(2))*p2x-m2*p2z*cos(q(2))*p2y+I432*cos(q(4)+q(3)+q(2))-m4*p4z*cos(q(4)+q(3)+q(2))*p4y+I331*sin(q(3)+q(2))+I332*cos(q(3)+q(2))-m3*p3z*sin(q(2))*a2-m4*p4z*sin(q(3)+q(2))*a3-m4*p4z*sin(q(4)+q(3)+q(2))*p4x-m4*p4z*sin(q(2))*a2

M(2,2) = m2*p2y^2+m2*p2x^2+m3*a2^2+m3*p3y^2+m3*p3x^2+I433+I233-2*m3*p3y*a2*sin(q(3))+2*m3*p3x*a2*cos(q(3))+I333+m4*a2^2+m4*p4x^2+m4*a3^2+m4*p4y^2+2*m4*a3*a2*cos(q(3))-2*m4*p4y*a3*sin(q(4))+2*m4*p4x*a3*cos(q(4))+2*m4*p4x*a2*cos(q(4)+q(3))-2*m4*p4y*a2*sin(q(4)+q(3))

M(2,3) = m3*p3y^2+m3*a2*p3x*cos(q(3))+I433+m4*a2*p4x*cos(q(4)+q(3))-2*m4*p4y*a3*sin(q(4))+2*m4*p4x*a3*cos(q(4))+m4*p4y^2+m4*a3^2+m4*p4x^2-m4*a2*p4y*sin(q(4)+q(3))+m4*a2*a3*cos(q(3))+I333+m3*p3x^2-m3*a2*p3y*sin(q(3))

M(2,4) = m4*p4x^2+I433+m4*a3*p4x*cos(q(4))-m4*a3*p4y*sin(q(4))+m4*p4y^2+m4*a2*p4x*cos(q(4)+q(3))-m4*a2*p4y*sin(q(4)+q(3))

M(3,1) = I431*sin(q(4)+q(3)+q(2))+I432*cos(q(4)+q(3)+q(2))-m4*p4z*sin(q(3)+q(2))*a3+I332*cos(q(3)+q(2))-m4*p4z*cos(q(4)+q(3)+q(2))*p4y-m4*p4z*sin(q(4)+q(3)+q(2))*p4x-m3*p3z*sin(q(3)+q(2))*p3x-m3*p3z*cos(q(3)+q(2))*p3y+I331*sin(q(3)+q(2))

M(3,2) = -m3*a2*p3y*sin(q(3))+m3*p3x^2+m3*p3y^2+I433+I333+m4*p4x^2+m3*a2*p3x*cos(q(3))-2*m4*p4y*a3*sin(q(4))+m4*p4y^2+m4*a3^2+2*m4*p4x*a3*cos(q(4))-m4*a2*p4y*sin(q(4)+q(3))+m4*a2*a3*cos(q(3))+m4*a2*p4x*cos(q(4)+q(3))

M(3,3) = m4*p4y^2+m4*p4x^2+m4*a3^2+2*m4*p4x*a3*cos(q(4))-2*m4*p4y*a3*sin(q(4))+I433+I333+m3*p3x^2+m3*p3y^2

M(3,4) = m4*a3*p4x*cos(q(4))-m4*a3*p4y*sin(q(4))+I433+m4*p4x^2+m4*p4y^2

M(4,1) = -m4*p4z*sin(q(4)+q(3)+q(2))*p4x-m4*p4z*cos(q(4)+q(3)+q(2))*p4y+I432*cos(q(4)+q(3)+q(2))+I431*sin(q(4)+q(3)+q(2))

M(4,2) = m4*p4x^2+I433-m4*a3*p4y*sin(q(4))+m4*p4y^2+m4*a2*p4x*cos(q(4)+q(3))+m4*a3*p4x*cos(q(4))-m4*a2*p4y*sin(q(4)+q(3))

M(4,3) = m4*p4y^2+m4*p4x^2+m4*a3*p4x*cos(q(4))-m4*a3*p4y*sin(q(4))+I433

M(4,4) = m4*p4y^2+I433+m4*p4x^2

V(1) = (-2*m3*p3x*a1*sin(q(3)+q(2))-2*m4*a2*a1*sin(q(2))+I421*cos(2*q(4)+2*q(3)+2*q(2))+I411*sin(2*q(4)+2*q(3)+2*q(2))+I412*cos(2*q(4)+2*q(3)+2*q(2))-I422*sin(2*q(4)+2*q(3)+2*q(2))-2*m4*p4x*a2*sin(q(4)+q(3)+2*q(2))-2*m2*cos(q(2))*p2y*a1-2*m3*sin(q(2))*a2*a1-2*m3*p3y*a2*cos(q(3)+2*q(2))-2*m3*p3x*a2*sin(q(3)+2*q(2))-2*m3*p3x*p3y*cos(2*q(3)+2*q(2))-2*m2*p2x*p2y*cos(2*q(2))-2*m3*p3y*a1*cos(q(3)+q(2))-2*m4*p4x*a1*sin(q(4)+q(3)+q(2))-2*m4*p4y*a1*cos(q(4)+q(3)+q(2))-2*m4*a3*a1*sin(q(3)+q(2))-2*m4*p4x*a3*sin(q(4)+2*q(3)+2*q(2))-2*m2*sin(q(2))*p2x*a1-2*m4*a3*a2*sin(q(3)+2*q(2))-2*m4*p4x*p4y*cos(2*q(4)+2*q(3)+2*q(2))-2*m4*p4y*a2*cos(q(4)+q(3)+2*q(2))-2*m4*p4y*a3*cos(q(4)+2*q(3)+2*q(2))+I312*cos(2*q(3)+2*q(2))+I221*cos(2*q(2))-I322*sin(2*q(3)+2*q(2))+I211*sin(2*q(2))-I222*sin(2*q(2))+I212*cos(2*q(2))+I311*sin(2*q(3)+2*q(2))-m4*a3^2*sin(2*q(3)+2*q(2))-m4*p4x^2*sin(2*q(4)+2*q(3)+2*q(2))-m4*a2^2*sin(2*q(2))+m4*p4y^2*sin(2*q(4)+2*q(3)+2*q(2))-m3*a2^2*sin(2*q(2))-m3*p3x^2*sin(2*q(3)+2*q(2))+I321*cos(2*q(3)+2*q(2))+m3*p3y^2*sin(2*q(3)+2*q(2))-m2*p2x^2*sin(2*q(2))+m2*p2y^2*sin(2*q(2)))*qd(1)*qd(2)+(-m4*p4x*a2*sin(q(4)+q(3))-2*m3*p3x*a1*sin(q(3)+q(2))+I421*cos(2*q(4)+2*q(3)+2*q(2))+I411*sin(2*q(4)+2*q(3)+2*q(2))+I412*cos(2*q(4)+2*q(3)+2*q(2))-I422*sin(2*q(4)+2*q(3)+2*q(2))-m4*p4x*a2*sin(q(4)+q(3)+2*q(2))-m3*p3y*a2*cos(q(3)+2*q(2))-m3*p3x*a2*sin(q(3)+2*q(2))-2*m3*p3x*p3y*cos(2*q(3)+2*q(2))-m3*p3y*a2*cos(q(3))-2*m3*p3y*a1*cos(q(3)+q(2))-2*m4*p4x*a1*sin(q(4)+q(3)+q(2))-2*m4*p4y*a1*cos(q(4)+q(3)+q(2))-2*m4*a3*a1*sin(q(3)+q(2))-2*m4*p4x*a3*sin(q(4)+2*q(3)+2*q(2))-m4*a3*a2*sin(q(3)+2*q(2))-2*m4*p4x*p4y*cos(2*q(4)+2*q(3)+2*q(2))-m4*p4y*a2*cos(q(4)+q(3)+2*q(2))-2*m4*p4y*a3*cos(q(4)+2*q(3)+2*q(2))+I312*cos(2*q(3)+2*q(2))-I322*sin(2*q(3)+2*q(2))-m4*p4y*a2*cos(q(4)+q(3))+I311*sin(2*q(3)+2*q(2))-m4*a3^2*sin(2*q(3)+2*q(2))-m4*p4x^2*sin(2*q(4)+2*q(3)+2*q(2))-m3*p3x*a2*sin(q(3))+m4*p4y^2*sin(2*q(4)+2*q(3)+2*q(2))-m3*p3x^2*sin(2*q(3)+2*q(2))+I321*cos(2*q(3)+2*q(2))+m3*p3y^2*sin(2*q(3)+2*q(2))-m4*a3*a2*sin(q(3)))*qd(1)*qd(3)+(-m4*p4x^2*sin(2*q(4)+2*q(3)+2*q(2))+m4*p4y^2*sin(2*q(4)+2*q(3)+2*q(2))-I422*sin(2*q(4)+2*q(3)+2*q(2))+I421*cos(2*q(4)+2*q(3)+2*q(2))+I411*sin(2*q(4)+2*q(3)+2*q(2))+I412*cos(2*q(4)+2*q(3)+2*q(2))-m4*p4x*a3*sin(q(4))-m4*p4y*a2*cos(q(4)+q(3))-m4*p4x*a3*sin(q(4)+2*q(3)+2*q(2))-m4*p4x*a2*sin(q(4)+q(3)+2*q(2))-m4*p4y*a3*cos(q(4))-m4*p4y*a3*cos(q(4)+2*q(3)+2*q(2))-m4*p4y*a2*cos(q(4)+q(3)+2*q(2))-m4*p4x*a2*sin(q(4)+q(3))-2*m4*p4x*p4y*cos(2*q(4)+2*q(3)+2*q(2))-2*m4*p4y*a1*cos(q(4)+q(3)+q(2))-2*m4*p4x*a1*sin(q(4)+q(3)+q(2)))*qd(1)*qd(4)+(-m3*p3z*cos(q(3)+q(2))*p3x+m3*p3z*sin(q(3)+q(2))*p3y-m3*p3z*cos(q(2))*a2-sin(q(3)+q(2))*I323+cos(q(3)+q(2))*I313-I423*sin(q(4)+q(3)+q(2))+I413*cos(q(4)+q(3)+q(2))-m4*p4z*cos(q(2))*a2-m2*p2z*cos(q(2))*p2x+m2*p2z*sin(q(2))*p2y+cos(q(2))*I213-sin(q(2))*I223-m4*p4z*cos(q(4)+q(3)+q(2))*p4x+m4*p4z*sin(q(4)+q(3)+q(2))*p4y-m4*p4z*cos(q(3)+q(2))*a3)*qd(2)^2+2*(-m3*p3z*cos(q(3)+q(2))*p3x+m3*p3z*sin(q(3)+q(2))*p3y-sin(q(3)+q(2))*I323+cos(q(3)+q(2))*I313-I423*sin(q(4)+q(3)+q(2))+I413*cos(q(4)+q(3)+q(2))-m4*p4z*cos(q(4)+q(3)+q(2))*p4x+m4*p4z*sin(q(4)+q(3)+q(2))*p4y-m4*p4z*cos(q(3)+q(2))*a3)*qd(2)*qd(3)+2*(-I423*sin(q(4)+q(3)+q(2))+I413*cos(q(4)+q(3)+q(2))-m4*p4z*cos(q(4)+q(3)+q(2))*p4x+m4*p4z*sin(q(4)+q(3)+q(2))*p4y)*qd(2)*qd(4)+(-m3*p3z*cos(q(3)+q(2))*p3x+m3*p3z*sin(q(3)+q(2))*p3y-sin(q(3)+q(2))*I323+cos(q(3)+q(2))*I313-I423*sin(q(4)+q(3)+q(2))+I413*cos(q(4)+q(3)+q(2))-m4*p4z*cos(q(4)+q(3)+q(2))*p4x+m4*p4z*sin(q(4)+q(3)+q(2))*p4y-m4*p4z*cos(q(3)+q(2))*a3)*qd(3)^2+2*(-I423*sin(q(4)+q(3)+q(2))+I413*cos(q(4)+q(3)+q(2))-m4*p4z*cos(q(4)+q(3)+q(2))*p4x+m4*p4z*sin(q(4)+q(3)+q(2))*p4y)*qd(3)*qd(4)+(-I423*sin(q(4)+q(3)+q(2))+I413*cos(q(4)+q(3)+q(2))-m4*p4z*cos(q(4)+q(3)+q(2))*p4x+m4*p4z*sin(q(4)+q(3)+q(2))*p4y)*qd(4)^2

V(2) = (1/2*m3*p3z*cos(q(3)+q(2))*p3x-1/2*m3*p3z*sin(q(3)+q(2))*p3y+1/2*sin(q(2))*I232-1/2*I431*cos(q(4)+q(3)+q(2))-1/2*cos(q(2))*I231+1/2*m2*p2z*cos(q(2))*p2x-1/2*m2*p2z*sin(q(2))*p2y+1/2*I432*sin(q(4)+q(3)+q(2))-1/2*m4*p4z*sin(q(4)+q(3)+q(2))*p4y-1/2*I331*cos(q(3)+q(2))+1/2*I332*sin(q(3)+q(2))+1/2*m3*p3z*cos(q(2))*a2+1/2*m4*p4z*cos(q(3)+q(2))*a3+1/2*m4*p4z*cos(q(4)+q(3)+q(2))*p4x+1/2*m4*p4z*cos(q(2))*a2)*qd(2)*qd(1)+(-2*m3*p3y*a2*cos(q(3))-2*m3*p3x*a2*sin(q(3))-2*m4*a3*a2*sin(q(3))-2*m4*p4x*a2*sin(q(4)+q(3))-2*m4*p4y*a2*cos(q(4)+q(3)))*qd(2)*qd(3)+(-2*m4*p4y*a3*cos(q(4))-2*m4*p4x*a3*sin(q(4))-2*m4*p4x*a2*sin(q(4)+q(3))-2*m4*p4y*a2*cos(q(4)+q(3)))*qd(2)*qd(4)+(-1/2*I431*cos(q(4)+q(3)+q(2))+1/2*I432*sin(q(4)+q(3)+q(2))+1/2*m4*p4z*cos(q(3)+q(2))*a3+1/2*I332*sin(q(3)+q(2))-1/2*m4*p4z*sin(q(4)+q(3)+q(2))*p4y+1/2*m4*p4z*cos(q(4)+q(3)+q(2))*p4x+1/2*m3*p3z*cos(q(3)+q(2))*p3x-1/2*m3*p3z*sin(q(3)+q(2))*p3y-1/2*I331*cos(q(3)+q(2)))*qd(3)*qd(1)+(-m3*p3x*a2*sin(q(3))-m4*p4x*a2*sin(q(4)+q(3))-m4*p4y*a2*cos(q(4)+q(3))-m4*a3*a2*sin(q(3))-m3*p3y*a2*cos(q(3)))*qd(3)^2+(-m4*p4x*a2*sin(q(4)+q(3))-2*m4*p4y*a3*cos(q(4))-2*m4*p4x*a3*sin(q(4))-m4*p4y*a2*cos(q(4)+q(3)))*qd(3)*qd(4)+(1/2*m4*p4z*cos(q(4)+q(3)+q(2))*p4x-1/2*m4*p4z*sin(q(4)+q(3)+q(2))*p4y+1/2*I432*sin(q(4)+q(3)+q(2))-1/2*I431*cos(q(4)+q(3)+q(2)))*qd(4)*qd(1)+(-m4*p4x*a2*sin(q(4)+q(3))-m4*p4y*a2*cos(q(4)+q(3)))*qd(4)*qd(3)+(-m4*p4x*a3*sin(q(4))-m4*p4y*a3*cos(q(4))-m4*p4x*a2*sin(q(4)+q(3))-m4*p4y*a2*cos(q(4)+q(3)))*qd(4)^2

V(3) = (-1/2*I431*cos(q(4)+q(3)+q(2))+1/2*I432*sin(q(4)+q(3)+q(2))+1/2*m4*p4z*cos(q(3)+q(2))*a3+1/2*I332*sin(q(3)+q(2))-1/2*m4*p4z*sin(q(4)+q(3)+q(2))*p4y+1/2*m4*p4z*cos(q(4)+q(3)+q(2))*p4x+1/2*m3*p3z*cos(q(3)+q(2))*p3x-1/2*m3*p3z*sin(q(3)+q(2))*p3y-1/2*I331*cos(q(3)+q(2)))*qd(3)*qd(1)+(1/2*m3*p3x*a2*sin(q(3))+1/2*m4*p4x*a2*sin(q(4)+q(3))+1/2*m4*p4y*a2*cos(q(4)+q(3))+1/2*m4*a3*a2*sin(q(3))+1/2*m3*p3y*a2*cos(q(3)))*qd(3)*qd(2)+(-2*m4*p4x*a3*sin(q(4))-2*m4*p4y*a3*cos(q(4)))*qd(3)*qd(4)+(1/2*m4*p4z*cos(q(4)+q(3)+q(2))*p4x-1/2*m4*p4z*sin(q(4)+q(3)+q(2))*p4y+1/2*I432*sin(q(4)+q(3)+q(2))-1/2*I431*cos(q(4)+q(3)+q(2)))*qd(4)*qd(1)+(1/2*m4*p4x*a2*sin(q(4)+q(3))+1/2*m4*p4y*a2*cos(q(4)+q(3)))*qd(4)*qd(2)+(-m4*p4x*a3*sin(q(4))-m4*p4y*a3*cos(q(4)))*qd(4)^2

V(4) = (1/2*m4*p4z*cos(q(4)+q(3)+q(2))*p4x-1/2*m4*p4z*sin(q(4)+q(3)+q(2))*p4y+1/2*I432*sin(q(4)+q(3)+q(2))-1/2*I431*cos(q(4)+q(3)+q(2)))*qd(4)*qd(1)+(1/2*m4*p4y*a3*cos(q(4))+1/2*m4*p4x*a2*sin(q(4)+q(3))+1/2*m4*p4x*a3*sin(q(4))+1/2*m4*p4y*a2*cos(q(4)+q(3)))*qd(4)*qd(2)+(1/2*m4*p4x*a3*sin(q(4))+1/2*m4*p4y*a3*cos(q(4)))*qd(4)*qd(3)

G(1) = 0

G(2) = m2*g*cos(q(2))*p2x-m2*g*sin(q(2))*p2y+m3*g*p3x*cos(q(3)+q(2))-m3*g*p3y*sin(q(3)+q(2))+m3*g*cos(q(2))*a2-m4*g*p4y*sin(q(4)+q(3)+q(2))+m4*g*a3*cos(q(3)+q(2))+m4*g*cos(q(2))*a2+m4*g*p4x*cos(q(4)+q(3)+q(2))

G(3) = m3*g*p3x*cos(q(3)+q(2))-m3*g*p3y*sin(q(3)+q(2))+m4*g*p4x*cos(q(4)+q(3)+q(2))-m4*g*p4y*sin(q(4)+q(3)+q(2))+m4*g*a3*cos(q(3)+q(2))

G(4) = m4*g*p4x*cos(q(4)+q(3)+q(2))-m4*g*p4y*sin(q(4)+q(3)+q(2))