ROBOTIC BACKHOE WITH HAPTIC DISPLAY

Students:	Joe Frankel, Matthew Kontz

Faculty & Staff:	Dr. Wayne Book.

Research goals:	Develop haptic controller for a John Deere backhoe, test and verify force estimation

Educational goals:	MSME degree

Active companies:	John Deere, Balluff, Georgia Hydraulic Cylinder, Wika, Sauer-Danfoss

Support:	John Deere, Balluff, Georgia Hydraulic Cylinder, Wika, and FPMC Center

Description:

Haptic user interfaces with force feedback are currently being developed for the control of hydraulic earthmoving equipment. This project involves integrating a haptic joystick, hydraulic cylinders with integrated position sensors, and low-cost electrohydraulic valves into a control system for a small commercial backhoe. Closed-loop haptic control of a single degree of freedom has been achieved on the dipperstick cylinder. The next phase will include fitting custom cylinders with pressure and internal position sensors and developing  closed-loop haptic control of all four degrees of freedom. Once the system is complete, a tip force estimation algorithm will be developed, tested, and verified.

Status as of 12/12:

- Single degree of freedom haptic control has been achieved on the dipperstick cylinder

- Valve system identification has been performed using the Hardware-In-The-Loop simulator, and a linear valve model has been developed.

- Kinematic and Dynamic models of the backhoe’s serial linkage have been developed

Work in Progress:

- Hydraulic cylinders with integrated position sensors are being fabricated

- A nonlinear valve model is being developed and verified with both HIL and 1-dof haptic testing data.

Future work:

- Fabrication and assembly of the 4-dof haptic system will be completed

- A tip force estimation algorithm using position and pressure feedback will be develop, tested, and evaluated.